# OpenCalc Project Documentation

Living hardware and firmware reference - updated 2026-05-23

# Purpose

This document is the working reference for the OpenCalc graphing calculator. It is meant to stay with the project and be updated as the firmware, PCB, enclosure, and feature set change.

# Project Summary

OpenCalc is an open source ESP32-S3 graphing calculator inspired by TI-84 style workflows. The current design targets a color ILI9341 LCD, touch input, a 5x10 physical button matrix, USB-C power/data, USB mass storage, Python-style scripts, optional Doom support, and LiPo battery operation.

# Current Repository Layout

| Path | Purpose |
| --- | --- |
| firmware/ | ESP-IDF firmware, storage images, components, build configuration, and firmware README. |
| firmware/main/ | Application entry point, config.h, board initialization, UI, math, scripting, Doom integration, USB storage code. |
| firmware/storage_image/ | Files flashed to the 8 MB FAT storage partition when Doom is included. Contains doom1.wad and scripts/fib.py. |
| firmware/storage_image_no_doom/ | Alternate storage image used when OPENCALC_FLASH_DOOM_WAD is 0. Contains scripts but no doom1.wad. |
| hardware/ | Graphic key layouts, PCB notes, and KiCad design files. |
| docs/ | Project documents and longer-form guides. |

# Current Firmware Status

- Target: ESP32-S3 using ESP-IDF v6.0.1.

- Display: ILI9341 over SPI2, 320x240 panel, firmware draws a 320x200 UI frame plus calculator chrome.

- Touch: XPT2046/TSC2046-style SPI touch support is initialized with interrupt handling.

- Buttons: 10 rows x 5 columns, interrupt-assisted scanning on the column pins.

- USB: TinyUSB mass storage exposes the FAT storage partition as an OpenCalc USB drive.

- Storage: custom 8 MB FAT partition at flash offset 0x410000.

- Apps/UI: 12-icon home menu, calculator, graph, table, Python scripts, settings, and Doom entry points.

- Wireless: WiFi and Bluetooth are configured off for battery savings.

# Build, Flash, and Monitor

A developer should clone the repository, install/source ESP-IDF, then build and flash from firmware/. Use idf.py flash rather than idf.py app-flash when the storage image should be written.

| Command | Use |
| --- | --- |
| idf.py set-target esp32s3 | Set the ESP-IDF target for a fresh checkout. |
| idf.py build | Build firmware, bootloader, partition table, and storage image. |
| idf.py flash | Flash bootloader, partition table, app, and storage image. |
| idf.py -p /dev/cu.usbmodemXXXX flash monitor | Flash and open serial monitor on macOS. |
| idf.py monitor | Open monitor after flashing. |

# Feature Toggles

| Define | Current Default | Meaning |
| --- | --- | --- |
| OPENCALC_ENABLE_BLUETOOTH | 0 | Bluetooth disabled for lower power draw. |
| OPENCALC_ENABLE_WIFI | 0 | WiFi disabled for lower power draw. |
| OPENCALC_ENABLE_DOOM | 1 | Doom code path available in firmware. |
| OPENCALC_FLASH_DOOM_WAD | 0 in current working tree | 0 uses storage_image_no_doom/; 1 uses storage_image/ with doom1.wad. Storage is flashed either way. |
| OPENCALC_USE_TRUE_POWER_OFF | 0 | 0 means software-off behavior; 1 is reserved for true latch power-off once hardware is ready. |

# Flash and Storage Layout

| Partition | Offset | Size | Purpose |
| --- | --- | --- | --- |
| nvs | 0x9000 | 0x5000 | NVS data. |
| phy_init | 0xe000 | 0x1000 | RF calibration partition. |
| factory | 0x10000 | 0x400000 | Main application. |
| storage | 0x410000 | 0x800000 | FAT filesystem exposed over USB mass storage. |

# USB Storage Behavior

The board should appear as an OpenCalc storage device when connected through the ESP32-S3 native USB port and firmware has initialized TinyUSB MSC. The COM/flashing port behavior depends on whether the board is in boot ROM download mode, app mode, or connected through a separate USB-UART interface on a dev board. The custom PCB should use one USB-C connector wired to native USB D+ and D-.

# Current Firmware Pin Map

These are the pins currently used by main/components/board_init.c. The PCB must match these or the firmware must be changed before manufacturing.

| Function | ESP32-S3 GPIO | Net / Signal |
| --- | --- | --- |
| USB D- | GPIO19 | USB_DM |
| USB D+ | GPIO20 | USB_DP |
| UART TX debug | GPIO43 | UART_TX optional test pad |
| UART RX debug | GPIO44 | UART_RX optional test pad |
| LCD SCLK | GPIO12 | LCD_SCLK |
| LCD MOSI | GPIO11 | LCD_MOSI |
| LCD MISO | GPIO13 | LCD_MISO |
| LCD DC | GPIO14 | LCD_DC |
| LCD reset | GPIO15 | LCD_RST |
| LCD CS | GPIO10 | LCD_CS |
| LCD backlight | GPIO46 | LCD_BL |
| Touch CS | GPIO41 | TOUCH_CS |
| Touch IRQ | GPIO42 | TOUCH_IRQ |

| Matrix Role | Pins |
| --- | --- |
| Rows | GPIO1, GPIO2, GPIO3, GPIO4, GPIO5, GPIO6, GPIO7, GPIO8, GPIO9, GPIO16 |
| Columns | GPIO17, GPIO18, GPIO38, GPIO39, GPIO40 |

# Key Matrix Logical Layout

| Row | Col0 | Col1 | Col2 | Col3 | Col4 |
| --- | --- | --- | --- | --- | --- |
| 0 | y= | window | zoom | trace | graph |
| 1 | 2nd | mode | stat | left | up |
| 2 | alpha | XthetaTn | del | down | right |
| 3 | math | []/[] | prgm | vars | clear |
| 4 | sqrt | sin | cos | tan | pi |
| 5 | ^2 | , | ( | ) | + |
| 6 | log | 7 | 8 | 9 | * |
| 7 | ln | 4 | 5 | 6 | - |
| 8 | sto | 1 | 2 | 3 | / |
| 9 | on (home) | 0 | . | (-) | enter |

# Display and Touch Hardware

- ILI9341 display is expected on SPI2 with separate LCD_CS and LCD_DC lines.

- Touch controller shares LCD SCLK/MOSI/MISO and uses TOUCH_CS plus TOUCH_IRQ.

- Backlight should be switched by a MOSFET/transistor if the LCD backlight current exceeds GPIO capability.

- LCD and touch connector pinout must be verified against the exact purchased module or FPC panel before PCB layout is finalized.

# Power System Requirements

- Single USB-C connector should provide power, native USB flashing, serial/monitor behavior, and USB mass storage.

- USB-C CC1 and CC2 each need a 5.1k pulldown to GND so a host recognizes the board as a USB device.

- USB D+ and D- should be routed through an ESD protection array close to the connector.

- A LiPo charger/power-path solution is needed so the board can run from USB or battery.

- The ON/OFF latch circuit is not yet enabled in firmware; the PCB can include the circuit now, but firmware pin defines must be updated when that circuit is chosen.

- If true hardware-off is used, firmware must assert the latch hold line immediately during boot before the user releases the ON button.

# Important PCB Guide V2 Review Findings

The uploaded PCB guide is directionally good, but it should not be treated as manufacturing-ready until the following issues are resolved.

| Severity | Issue | Required Action |
| --- | --- | --- |
| Blocking | The guide says 49 keys are in the matrix and the 50th ON/OFF key is not in the matrix, but current firmware maps on (home) as row 9, col 0 inside the 5x10 matrix. | Choose the electrical behavior. If ON must be latch-only, update firmware to read a separate power/home input. If ON must also be a normal key, design dual behavior carefully: latch/power function plus a firmware-readable signal when powered. |
| Blocking | Battery voltage divider suggestion uses GPIO7, but GPIO7 is already ROW6 in the current matrix. In fact the current pin map consumes all simple ADC-capable pins with matrix, SPI, or USB duties. | Do not connect battery ADC to GPIO7. Either remap pins to free an ADC-capable GPIO, add an external I2C fuel gauge/ADC, or omit battery percentage for the first PCB spin. |
| High | Power latch guide suggests GPIO45. GPIO45 is a strapping-sensitive pin on ESP32-S3 designs and can affect boot if pulled incorrectly. | Verify boot strap requirements and resistor values before using GPIO45, or select another free output-capable GPIO if available on the module. |
| High | LCD backlight is assigned to GPIO46. This may be acceptable depending on the exact ESP32-S3 module/pin capability, but it is also a boot/strap-sensitive area and should not directly drive LED current. | Verify GPIO46 against the exact module datasheet and use a MOSFET/backlight driver. If there is any doubt, move backlight control to a safer free GPIO and update firmware. |
| Medium | The guide includes optional microSD on the shared SPI bus, but the current product goal uses internal 8 MB FAT storage and flashed files. | Leave microSD unpopulated or optional unless the PCB has space and a clear use case. If added, assign CS to a confirmed free GPIO and update firmware later. |
| Medium | The guide references POWER_BUTTON_ENABLED/POWER_HOLD_ENABLED as firmware symbols, but current board_init.c keeps both disabled and NC. | Before relying on true power-off, add final firmware defines for the latch hold and optional button sense pins. |

# Recommended PCB Pre-Manufacturing Checklist

- Confirm exact ESP32-S3 module variant, flash size, PSRAM, antenna keepout, and exposed GPIOs.

- Confirm USB-C D+ to GPIO20 and D- to GPIO19, with CC pulldowns and ESD protection.

- Confirm EN and GPIO0 have pullups, reset/boot buttons or accessible test pads.

- Confirm all LCD/touch nets match the purchased display connector pinout.

- Confirm key matrix row/column nets match firmware and the physical key artwork.

- Resolve ON/OFF latch versus matrix/home behavior before PCB order.

- Resolve battery monitoring method before PCB order.

- Run KiCad ERC and DRC with zero unresolved errors.

- Review Gerbers in an external viewer before fabrication.

- Order first PCB spin in small quantity only.

# Firmware Architecture

| File / Component | Role |
| --- | --- |
| main/main.c | Application startup: storage, USB MSC, board init, UI loop. |
| main/config.h | Local feature toggles for WiFi, Bluetooth, Doom, storage image, and power-off behavior. |
| components/board_init.c | LCD, touch, keypad, sleep/power helper functions, drawing helpers. |
| components/opencalc_ui.c | Home menu, app screens, calculator input, graphing UI, scripts menu, settings, Doom toggle. |
| components/opencalc_math.c | Math parser/evaluator and graph function evaluation support. |
| components/storage.c | Mounts FAT storage partition for the app and sets volume label. |
| components/usb_msc.c | TinyUSB MSC setup and storage ownership switching. |
| components/tiny-python.c | Small embedded Python-like interpreter for scripts. |
| components/opencalc_doom.c | Doom integration and keypad mapping. |

# Calculator User Interface

- Home screen shows a 12-icon app grid.

- Top bar should include page/app identity and battery indicator as the UI matures.

- Arrow keys move selection. Enter opens. Touch should select icons or activate useful controls where implemented.

- Settings includes display, USB storage, wireless, power, and about entries.

# Current App Set

| App | Purpose |
| --- | --- |
| Calculator | Expression entry, numeric evaluation, history, variables. |
| Graph | Y= functions, plotting, trace, zoom, table connection. |
| Table | Function value table and table setup. |
| Python | Lists scripts from storage and runs selected scripts when interpreter is stable. |
| Statistics | Plots, regression, tests, distributions. |
| Lists | Named lists and list math. |
| Matrices | Matrix storage and operations. |
| Solver | Equation, polynomial, and system solving. |
| Finance | TVM, cash flow, amortization. |
| Conics | Conic templates and intersections. |
| Settings | Display, storage, wireless, power, about. |
| Doom | Optional game mode using doom1.wad. |

# Doom Support

- Doom is optional and controlled by OPENCALC_ENABLE_DOOM plus the presence of doom1.wad in storage.

- The shareware IWAD is expected at storage_image/doom1.wad when OPENCALC_FLASH_DOOM_WAD is 1.

- Current shortcut goal: alpha then 2nd toggles Doom on/off.

- Directional keys move; Y= is intended to shoot based on the latest key-layout decision.

# Known Risks and Open Decisions

- Final ON/OFF electrical behavior must be resolved before PCB manufacture.

- Battery monitoring needs a real pin/resource decision.

- The tiny Python interpreter has previously caused heap corruption when executing scripts; file reading and interpreter safety still need hardening.

- Doom memory and performance should be tested on the final ESP32-S3 module variant with PSRAM.

- USB storage ownership must remain stable between host access and app access.

- The final LCD module pinout and backlight current must be verified before committing PCB footprints.

# Manufacturing Notes

- Use test pads for 3V3, GND, VBAT, USB D+, USB D-, EN, GPIO0, UART TX/RX, and any power latch hold/sense nets.

- Keep the ESP32-S3 antenna area free of copper and components per module datasheet.

- Route USB D+/D- as a short, matched differential pair over continuous ground.

- Place USB ESD protection close to the connector.

- Keep battery area free of vias and sharp component leads if the LiPo sits against the PCB.

- Label battery polarity and boot/reset pads on silkscreen.

# Update Log

| Date | Update |
| --- | --- |
| 2026-05-23 | Initial living documentation generated from current repository state and PCB guide V2 review. |