CALCOPEN

The Open-Source TI-84 Reimagined

Product Planning Document Version 1.0 ¦ May 2026 ¦ Target MSRP: $25

ESP32-Based Scientific Calculator with USB-C, Custom Script Runtime & LCD Display

# 1. Executive Summary

CalcOpen is a modern reimagining of the TI-84 graphing calculator, built on the ESP32 microcontroller platform. It delivers every core mathematical function students and engineers expect from a TI-84, while adding USB-C charging and PC emulation, a high-contrast LCD display, and a clean MicroPython scripting environment — all at a $25 target retail price.

Goal: Build a fully open hardware/software graphing calculator that costs less than a TI-84 battery pack, runs custom scripts, and charges from any USB-C cable.

The device is designed for students, STEM educators, hardware hackers, and the maker community. All hardware design files and firmware will be released under open-source licenses.

# 2. Product Overview

## 2.1  Core Value Proposition

| Feature | TI-84 Plus CE | CalcOpen |
| --- | --- | --- |
| Retail Price | $119–$150 | $25 target |
| Processor | Z80 @ 6 MHz | ESP32 @ 240 MHz |
| Charging | 4× AAA batteries | USB-C rechargeable |
| Custom Scripts | TI-BASIC only | MicroPython + TI-BASIC compat |
| Connectivity | USB Mini-B (data only) | USB-C (charge + HID emulation) |
| Open Source | No | Full hardware + firmware |
| Wireless | None | Wi-Fi + Bluetooth (ESP32) |

## 2.2  Target Users

- High school and college students needing a capable graphing calculator

- STEM educators wanting an affordable classroom-scale deployment

- Embedded/hardware hobbyists who want a hackable calculator

- Developers interested in building custom scientific tools

# 3. Hardware Design

## 3.1  Processor — ESP32

The ESP32 (Espressif Systems) is the heart of CalcOpen. It offers far more compute power than the Z80 in a TI-84 while remaining inexpensive and well-supported by the open-source community.

| Specification | Value |
| --- | --- |
| Chip | ESP32-S3 (recommended) or ESP32 WROOM-32 |
| CPU Cores | Dual-core Xtensa LX7 @ up to 240 MHz |
| Flash | 4 MB internal + 8 MB PSRAM (S3 variant) |
| RAM | 512 KB SRAM + 8 MB PSRAM |
| Wi-Fi | 802.11 b/g/n (2.4 GHz) |
| Bluetooth | BLE 5.0 |
| GPIO | 45 pins (sufficient for keypad matrix + display + USB) |
| Unit Cost (bulk 500) | ~$2.80–$3.50 |

The ESP32-S3 is strongly recommended over classic ESP32 because it includes native USB-OTG support, enabling the USB-C HID emulation (PC keyboard/device) feature without a secondary USB bridge chip.

## 3.2  Display — LCD Screen

A high-contrast monochrome or 16-bit color LCD provides the primary visual output. Two viable options are evaluated:

| Option | Module | Resolution | Interface | Cost | Recommendation |
| --- | --- | --- | --- | --- | --- |
| A (Preferred) | ILI9341 TFT LCD | 320×240 px, 2.8" | SPI | ~$3.00 | ✓ Best value |
| B (Alternate) | ST7789 TFT LCD | 240×240 px, 1.54" | SPI | ~$2.50 | Smaller form factor |
| C (Budget) | UC1701 Mono LCD | 128×64 px, 1.3" | SPI/I2C | ~$1.50 | Lower power draw |

Option A (ILI9341, 320×240) is the recommended choice. It closely matches the TI-84's display area, supports color graph rendering, and is widely available from multiple suppliers. It directly connects to the ESP32 via SPI at up to 40 MHz.

## 3.3  USB-C — Charging & PC Emulation

This is one of CalcOpen's most differentiating features. The ESP32-S3's native USB-OTG peripheral enables two major capabilities from a single USB-C port:

### 3.3.1  USB-C Charging

- A TP4056 or BQ24079 lithium battery charge controller manages charging from the USB-C 5V rail

- 1,800–2,000 mAh LiPo cell provides approximately 20–30 hours of typical use

- Charging indicator LED (red = charging, green = full)

- Over-voltage and over-temperature protection included in charge IC

### 3.3.2  PC Emulation (HID + Mass Storage)

- When plugged into a PC or laptop, the ESP32-S3 enumerates as a USB HID device AND a USB Mass Storage device simultaneously

- HID mode: The calculator can function as a USB keyboard, sending keystrokes to the host — useful for data entry workflows

- Mass Storage mode: The calculator's internal flash partition appears as a USB drive, allowing drag-and-drop script upload without any special software

- CDC mode (optional): Virtual COM port for serial debugging and script REPL access via a terminal emulator

## 3.4  Keypad Matrix

The keypad replicates the TI-84 Plus CE layout: 10 columns × 10 rows = 96 key positions, handled by a GPIO matrix scan. A dedicated keypad scanning firmware task runs at 5 ms intervals to ensure zero missed keypresses even during heavy computation.

- Keypad ICs: Direct ESP32 GPIO scan (no additional IC needed for ≤ 100 keys)

- Key travel: 0.3 mm tactile domes (ø 6 mm), rated 100,000 actuations

- Layout: Mirrors TI-84 Plus CE — function keys, numpad, graphing keys, ALPHA/2ND modifiers

## 3.5  Power System

| Component | Spec | Est. Cost |
| --- | --- | --- |
| LiPo Battery | 3.7V, 2000 mAh, 50 × 34 × 5 mm | $2.20 |
| Charge IC | TP4056 with DW01A protection | $0.30 |
| 3.3V Regulator | AMS1117-3.3 LDO | $0.15 |
| Power Switch | SPDT slide switch | $0.10 |
| Battery Life (est.) | ~25 hrs typical use | — |

## 3.6  Bill of Materials (BOM) Summary

Target unit cost at 500-unit production run:

| Component | Unit Cost |
| --- | --- |
| ESP32-S3 module | $3.20 |
| ILI9341 2.8" LCD | $3.00 |
| LiPo 2000 mAh | $2.20 |
| TP4056 charge IC + protection | $0.30 |
| PCB (2-layer, JLCPCB) | $1.80 |
| Keypad dome sheet (96 keys) | $1.50 |
| Enclosure (injection mold amortized) | $4.00 |
| USB-C connector | $0.25 |
| Passive components (caps, resistors, LEDs) | $0.80 |
| Assembly labor (PCBA) | $2.50 |
| Miscellaneous (cables, box, sticker) | $1.20 |
| TOTAL COGS (500-unit run) | ~$20.75 |
| Target MSRP | $25.00 |
| Gross Margin | ~17% |

At 1,000+ units, COGS drops to ~$17–$18 due to MOQ breaks on the ESP32 module, PCB, and enclosure mold amortization, improving margin to ~28–32%.

# 4. Firmware & Software Architecture

## 4.1  Language Strategy: C Core with Embedded MicroPython

CalcOpen uses a two-layer language architecture: the entire firmware, math engine, and hardware drivers are written in C, while MicroPython is embedded as a C library to provide the user-facing scripting runtime. This is the ideal split for a resource-constrained embedded device. You get bare-metal performance where it matters and a friendly, familiar scripting environment on top.

The entire calculator engine, display driver, keypad scanner, and USB stack are pure C. MicroPython runs as a single FreeRTOS task hosted by the C firmware and calls back into C functions via a custom built-in module.

### Why C for the Core?

- Deterministic, low-latency keypad scanning and display rendering with no garbage collector pauses

- Direct register-level access to ESP32-S3 peripherals: SPI, USB-OTG, ADC, GPIO

- Full control over FreeRTOS task priorities and stack sizes

- Math engine can use fixed-point and IEEE 754 double arithmetic with predictable cycle counts

- Smallest possible binary footprint, critical when sharing 4 MB flash with MicroPython and user scripts

### Why MicroPython for Scripting?

- Users write scripts in standard Python with no new language to learn

- MicroPython is designed to be embedded inside a C application; Espressif maintains an official ESP-IDF port

- The calc built-in module (written in C) is registered with MicroPython at compile time, making all calculator functions available to Python scripts as native calls

- MicroPython's REPL is available over the USB-C serial port for live development

- ~170 KB flash footprint, very manageable on 4 MB flash

### Project File Structure

| File / Directory | Language | Description |
| --- | --- | --- |
| main/keypad_task.c | C | GPIO matrix scan every 5 ms, debounce, key event queue |
| main/display_driver.c | C | SPI driver for ILI9341, framebuffer, text/graphics primitives |
| main/math_engine.c | C | All TI-84 math: arithmetic, graphing, stats, matrices, solver |
| main/usb_hid.c | C | ESP32-S3 USB-OTG: HID keyboard, mass storage, CDC serial |
| main/power_mgmt.c | C | Battery ADC monitoring, sleep/wake, charge state LED |
| main/micropython_task.c | C | FreeRTOS task that hosts the MicroPython runtime |
| main/calc_module.c | C | MicroPython built-in module: bridges Python calls to C engine |
| scripts/*.py | MicroPython | User scripts stored on flash FAT32 partition |

### How MicroPython Embeds into C

MicroPython is added to the ESP-IDF project as a component. The micropython_task.c file initializes the runtime and registers the calc module:

| Step | Code / Action |
| --- | --- |
| 1. Init runtime | mp_embed_init(heap_buf, HEAP_SIZE, stack_top) |
| 2. Register calc module | Declare MP_DEFINE_CONST_FUN_OBJ for each C function, add to module globals table |
| 3. Run a script | mp_embed_exec_str(python_source_code) called from C with any .py string |
| 4. Call C from Python | import calc then calc.eval('2+2') routes to math_engine.c |
| 5. REPL access | USB CDC serial port: connect with any terminal for a live Python prompt |

### calc_module.c Bridge (simplified)

This C file is the bridge between Python scripts and the C math engine. Each function is registered as a MicroPython built-in:

| Python call (user script) | C function in calc_module.c | Calls into |
| --- | --- | --- |
| calc.eval('sin(pi/2)') | mp_obj_t calc_eval(mp_obj_t expr) | math_engine_eval() |
| calc.graph('x^2') | mp_obj_t calc_graph(mp_obj_t func) | display_graph_function() |
| calc.display.print(x, y, s) | mp_obj_t calc_display_print(...) | display_draw_text() |
| calc.keypad.read() | mp_obj_t calc_keypad_read(void) | keypad_wait_event() |
| calc.usb.hid_send(key) | mp_obj_t calc_usb_hid_send(mp_obj_t k) | usb_hid_send_key() |

## 4.2  FreeRTOS Task Architecture

All tasks are written in C. The MicroPython task is one peer among equals and cannot starve hardware tasks because keypad scanning and display rendering run at higher priority:

| FreeRTOS Task | Priority | Stack | Description |
| --- | --- | --- | --- |
| keypad_scan_task | High | 2 KB | Polls GPIO matrix every 5 ms, debounces, queues key events |
| display_render_task | Medium-High | 8 KB | Composites UI frames, drives SPI to LCD at up to 30 FPS |
| calc_engine_task | Medium | 32 KB | Handles all mathematical computation and state machine |
| micropython_task | Medium | 64 KB | Hosts MicroPython runtime, runs user .py scripts |
| usb_hid_task | High | 4 KB | Manages USB-C HID, CDC, and mass storage enumeration |
| power_mgmt_task | Low | 2 KB | Monitors battery ADC, manages sleep/wake states |

## 4.3  Calculator Engine

The core math engine implements 100% of TI-84 Plus CE functionality:

### Arithmetic & Algebra

- Integer, fixed-point, and floating-point arithmetic (IEEE 754 double precision)

- Complex number support

- Fraction simplification and exact arithmetic mode

- Symbolic algebra: factor, expand, solve (via lightweight CAS library)

### Graphing

- Function graphing: y= editor with up to 10 simultaneous functions

- Parametric, polar, and sequence graphing modes

- Trace, zoom, and intercept/root/max/min/integral analysis tools

- Table view for any graphed function

### Statistics & Data

- 1-Var and 2-Var statistics: mean, median, std dev, variance, quartiles

- Regression: linear, quadratic, cubic, exponential, logarithmic, sinusoidal, logistic

- Probability distributions: normal, binomial, Poisson, t-distribution, chi-square, F

- Hypothesis tests: z-test, t-test, χ²-test, ANOVA

### Linear Algebra & Calculus

- Matrix operations: add, multiply, inverse, determinant, RREF, eigenvalues

- Numeric differentiation (dy/dx) and definite integration (∫f dx) on graphed functions

- Solver app: solve any single-variable equation numerically

### Finance & Conversion

- TVM Solver: PV, FV, PMT, I%, N (matches TI-84 Finance app)

- Unit conversions (length, mass, volume, temperature, speed)

## 4.4  Custom Script Runtime (MicroPython)

This is CalcOpen's flagship differentiator. A full MicroPython interpreter runs as a FreeRTOS task, giving users a familiar, powerful scripting environment.

### Script Storage & Management

- Scripts stored in a FAT32 partition on the ESP32-S3's internal flash (1 MB dedicated)

- When connected via USB-C, the partition mounts as a USB drive — drag and drop .py files directly from any PC or Mac

- On-device script browser: press [APPS] → Scripts to see, select, and run any stored script

- Scripts can be edited on-device with a minimal line editor or via the USB serial REPL

### MicroPython API (calc module)

A built-in calc Python module exposes the calculator engine to scripts:

| Function | Description |
| --- | --- |
| calc.eval(expr) | Evaluate a math expression string, returns float |
| calc.graph(func_str) | Graph a function string on the display |
| calc.display.print(x, y, text) | Draw text at pixel coordinates |
| calc.display.clear() | Clear the screen |
| calc.keypad.read() | Blocking read of next keypress, returns key code |
| calc.matrix(rows, cols, data) | Create a matrix object for linear algebra |
| calc.stats.linreg(x_list, y_list) | Run linear regression, returns slope/intercept |
| calc.usb.hid_send(keycode) | Send a keystroke to the connected PC (HID mode) |

Example use case: A student writes a Python script that reads sensor data over Bluetooth (ESP32 BLE), plots it live on the LCD, and sends summary statistics to their laptop via USB HID — none of this is possible on a stock TI-84.

## 4.5  TI-84 Compatibility

- TI-BASIC interpreter included — run existing TI-84 programs with no modification

- File format compatibility: .8xp program files can be loaded via USB drag-and-drop

- Same menu structure and key bindings as TI-84 Plus CE for zero learning curve

# 5. Physical & Mechanical Design

## 5.1  Form Factor

CalcOpen closely follows the TI-84 Plus CE physical profile to feel immediately familiar to existing users.

| Dimension | TI-84 Plus CE | CalcOpen Target |
| --- | --- | --- |
| Height | 190 mm | 192 mm |
| Width | 76 mm | 76 mm |
| Depth | 19 mm | 16 mm (slimmer — no battery bay) |
| Weight | 218 g (with batteries) | 165 g (LiPo, no batteries) |
| Screen Area | 83 × 56 mm visible | 57.6 × 43.2 mm (ILI9341) |

## 5.2  Enclosure

- Two-piece ABS plastic shell (top + bottom), snap-fit with 4 × M2 screws

- Front face: matte black with key labels printed using UV pad printing

- USB-C port: bottom edge (matching phone convention — natural for desk charging)

- Slide power switch: right side

- Charging LED indicator: front face, near top-right corner

- Wrist strap anchor: top edge

## 5.3  PCB Layout

- 2-layer PCB, 130 × 65 mm

- ESP32-S3 module top-center

- LCD connector (ZIF/FFC) top edge

- TP4056 and power rails bottom-left

- Keypad matrix connector (20-pin FFC) bottom-right

- USB-C receptacle bottom edge, centered

# 6. Development Roadmap

| Phase | Milestone | Duration | Deliverable |
| --- | --- | --- | --- |
| Phase 0 | Requirements & Research | 2 weeks | BOM finalized, schematic started |
| Phase 1 | Prototype PCB v0.1 | 4 weeks | Breadboard/dev-kit proof of concept |
| Phase 2 | Firmware Core | 6 weeks | Calc engine + display + keypad working |
| Phase 3 | MicroPython Integration | 4 weeks | Script runtime + USB mass storage |
| Phase 4 | USB-C HID + Charging | 3 weeks | Full USB-C feature set validated |
| Phase 5 | PCB v1.0 + Enclosure | 5 weeks | Assembled prototype in final shell |
| Phase 6 | Alpha Test | 3 weeks | 10-unit alpha build, bug fixes |
| Phase 7 | Beta / Pre-production | 4 weeks | 50-unit beta, regulatory (FCC/CE) |
| Phase 8 | Production Run 1 | 6 weeks | 500-unit production + fulfillment |

Total estimated timeline from kickoff to first production units: approximately 37 weeks (~9 months).

# 7. Open Source Strategy

## 7.1  Licensing

| Component | License | Rationale |
| --- | --- | --- |
| Hardware schematics & PCB files | CERN OHL-W v2 | Copyleft, allows commercial use with attribution |
| Firmware (ESP-IDF application) | MIT | Maximum permissiveness, encourages forks |
| MicroPython calc module | MIT | Compatible with MicroPython's MIT license |
| Documentation | CC BY 4.0 | Free to share and adapt with attribution |
| Enclosure 3D files | CC BY-SA 4.0 | ShareAlike encourages community variants |

## 7.2  Community & Ecosystem

- GitHub repository: hardware, firmware, documentation in mono-repo

- Script library: community-contributed MicroPython scripts (physics, chemistry, engineering)

- Forum/Discord: bug reports, feature requests, script sharing

- KiCad files published so anyone can manufacture their own PCB

# 8. Risks & Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| ESP32-S3 USB-OTG stability issues | Medium | High | Prototype early; fallback to CH340/FTDI bridge chip |
| LCD cost increase from supply constraints | Low | Medium | Pre-qualify 3 display suppliers with same SPI interface |
| MicroPython memory pressure (512 KB SRAM) | Medium | Medium | Use PSRAM for heap; profile early; cull unused modules |
| Enclosure mold cost exceeds budget | Medium | High | Use Hubs/Xometry for first run; 3D-printed beta cases |
| FCC/CE certification delays | Low | Medium | Use pre-certified ESP32 module; hire consultant for EMC |
| TI trademark/IP concerns | Low | Low | No TI trademarks used; clean-room calc engine implementation |
| $25 MSRP infeasible at <500 units | Medium | High | Crowdfund to guarantee 500+ unit MOQ before committing |

# 9. Success Metrics

| Metric | Target |
| --- | --- |
| Unit cost at 500-unit run | ≤ $21.00 COGS |
| Battery life | ≥ 20 hours typical use |
| Calculator cold boot time | < 2 seconds to ready state |
| TI-BASIC compatibility | ≥ 95% of common TI-84 programs run correctly |
| Script load time (100-line .py) | < 500 ms |
| USB-C enumeration time | < 3 seconds on Windows/Mac/Linux |
| Keypad scan latency | ≤ 10 ms end-to-end |
| Community scripts at launch | ≥ 20 example scripts in repo |

# 10. Appendix — Useful References

- ESP32-S3 Technical Reference Manual: https://www.espressif.com/sites/default/files/documentation/esp32-s3_technical_reference_manual_en.pdf

- MicroPython ESP32 port: https://micropython.org/download/ESP32_GENERIC/

- ILI9341 Datasheet: available from Adafruit and multiple display suppliers

- TP4056 Lithium Charger IC: HN0T-TP4056 datasheet

- KiCad EDA (free PCB design): https://www.kicad.org

- JLCPCB PCB + PCBA service: https://jlcpcb.com

- TI-84 Plus CE hardware guide (community reverse engineering): TI-Planet.org

This document is a living planning artifact. Update version number and date on any substantive revision. All cost estimates are as of May 2026 and should be re-validated before production commitment.

# 11. Getting Started — Full Step-by-Step Checklist

Complete these steps in order. Each group builds on the last. Do not skip ahead to PCB design until the breadboard prototype is fully validated.

## Group A — Admin & Setup (Week 1)

| # | Step | Done? |
| --- | --- | --- |
| A1 | Create a GitHub repository named 'calcopen' (or your chosen name). Initialize with a README, MIT license, and .gitignore for C/ESP-IDF projects. | [ ] |
| A2 | Commit this planning document to the repo under /docs. | [ ] |
| A3 | Install ESP-IDF on your development machine. Follow the official Espressif getting-started guide for your OS (Windows/Mac/Linux). Confirm 'idf.py --version' runs successfully. | [ ] |
| A4 | Install KiCad 7+ for schematic and PCB design. Free download at kicad.org. | [ ] |
| A5 | Create accounts on JLCPCB and Mouser/DigiKey for future PCB ordering and component sourcing. | [ ] |
| A6 | Order 2x ESP32-S3 DevKit boards with pins (e.g. ESP32-S3-DevKitC-1) from Amazon or AliExpress. ~$10 each. These are your breadboard prototyping boards. | [ ] |
| A7 | Order 1x ILI9341 2.8-inch TFT LCD SPI module. ~$8 on Amazon. Make sure it is the SPI variant, not I2C. | [ ] |
| A8 | Order a breadboard, jumper wires, and a handful of 6mm tactile push buttons (10-20 pcs). | [ ] |

## Group B — First Firmware (Week 2)

| # | Step | Done? |
| --- | --- | --- |
| B1 | Flash MicroPython onto one ESP32-S3 DevKit. Download the .bin from micropython.org and flash with 'esptool.py'. Confirm you get a Python REPL over USB serial. | [ ] |
| B2 | Wire the ILI9341 LCD to the ESP32-S3 DevKit over SPI (5 wires: MOSI, SCLK, CS, DC, RST). Use a pinout diagram from Adafruit or Espressif docs. | [ ] |
| B3 | Write a minimal ESP-IDF C app (not MicroPython yet) that initializes the SPI bus and draws a filled rectangle on the LCD. Confirm pixels appear. | [ ] |
| B4 | Add text rendering: draw the string 'CalcOpen' on screen. This validates your display_driver.c foundation. | [ ] |
| B5 | Wire 5 tactile buttons to GPIO pins on the DevKit. Write a C task that scans them every 5 ms and prints the key number over serial. This is your keypad_scan_task.c skeleton. | [ ] |
| B6 | Commit all working code to GitHub under /firmware. | [ ] |

## Group C — USB-C Validation (Week 2–3)

| # | Step | Done? |
| --- | --- | --- |
| C1 | Create a new ESP-IDF project targeting USB-OTG. Flash the Espressif 'tusb_hid_device' example. Plug into your laptop and confirm the ESP32-S3 enumerates as a USB HID device in Device Manager / System Information. | [ ] |
| C2 | Flash the Espressif 'tusb_msc_sdmmc' or 'tusb_msc_spiflash' example. Confirm the ESP32-S3 appears as a USB drive on your laptop. Create a test.txt file on it. | [ ] |
| C3 | If both C1 and C2 succeed: USB-C emulation is validated. Document the working example project names and IDF version in your repo README. | [ ] |
| C4 | If either fails: check that you are using ESP32-S3 (not classic ESP32) and that your IDF version is 5.0+. If still failing, evaluate adding a CH340 USB-serial bridge as fallback (see Risk table in Section 8). | [ ] |

## Group D — Embed MicroPython into C (Week 3–4)

| # | Step | Done? |
| --- | --- | --- |
| D1 | Clone the MicroPython repository. Check out the esp32 port and confirm it builds with ESP-IDF using the instructions in ports/esp32/README.md. | [ ] |
| D2 | Integrate MicroPython as a component inside your main ESP-IDF project by adding it as a git submodule under /components/micropython. | [ ] |
| D3 | Create micropython_task.c: a FreeRTOS task that calls mp_embed_init() and then mp_embed_exec_str() to run a hardcoded Python string like 'print(1+1)'. Confirm output appears over serial. | [ ] |
| D4 | Create calc_module.c: register at least one function, e.g. calc.add(a, b), that calls a C function and returns the result to Python. Test it by running 'import calc; print(calc.add(3, 4))' via the REPL. | [ ] |
| D5 | Extend calc_module.c with calc.display.print(x, y, text) that calls your display_driver.c. Run a Python script that draws text on the LCD. This is the first end-to-end demo. | [ ] |
| D6 | Commit and tag this milestone in GitHub as v0.1-proof-of-concept. | [ ] |

## Group E — Math Engine (Week 4–7)

| # | Step | Done? |
| --- | --- | --- |
| E1 | Implement math_engine.c: start with basic arithmetic (+, -, *, /), then trig (sin, cos, tan), then logarithms and exponentials. Use C's math.h for underlying functions. | [ ] |
| E2 | Write a simple expression parser in C (recursive descent parser is recommended). It should take a string like 'sin(pi/2)+3' and return a double. | [ ] |
| E3 | Expose the parser as calc.eval(expr) in calc_module.c. Write a Python test script that evaluates 20 expressions and prints results. Verify against known answers. | [ ] |
| E4 | Implement the graphing engine: a C function that takes a function string, evaluates it at N x-values across a range, and plots the pixel results on the LCD framebuffer. | [ ] |
| E5 | Implement matrix operations: allocate a 2D double array, implement multiply, transpose, and determinant in C. Expose as calc.matrix() in Python. | [ ] |
| E6 | Implement basic statistics: mean, std dev, linear regression over a list of doubles passed from Python. | [ ] |
| E7 | Add TI-BASIC interpreter skeleton: parse and execute simple variable assignment, arithmetic, and Disp statements. Full compatibility can come later. | [ ] |

## Group F — Full Keypad & UI (Week 7–9)

| # | Step | Done? |
| --- | --- | --- |
| F1 | Design the full 10x10 keypad matrix schematic in KiCad. Map every TI-84 key to an ESP32-S3 GPIO pair. | [ ] |
| F2 | Wire a larger test keypad (at least 20 keys) on your breadboard and validate the matrix scan handles simultaneous keypresses without ghosting. Add diodes if needed. | [ ] |
| F3 | Implement the calculator UI state machine in C: home screen, y= editor, graph view, stats menu, matrix editor. Drive it entirely from keypad events. | [ ] |
| F4 | Implement the script browser UI: list .py files from the FAT32 flash partition, allow selection with arrow keys, run on ENTER. | [ ] |

## Group G — PCB Design (Week 9–12)

| # | Step | Done? |
| --- | --- | --- |
| G1 | Complete the full KiCad schematic: ESP32-S3-WROOM-1 (bare module, no pins), ILI9341 LCD FFC connector, 10x10 keypad matrix with diodes, TP4056 charge circuit, LiPo connector, USB-C receptacle, AMS1117-3.3 regulator, power switch, LEDs. | [ ] |
| G2 | Run ERC (Electrical Rules Check) in KiCad. Fix all errors and warnings. | [ ] |
| G3 | Lay out the 2-layer PCB: 130x65 mm. Follow the component placement described in Section 5.3. | [ ] |
| G4 | Run DRC (Design Rules Check) for JLCPCB's 2-layer standard rules (min trace 0.1 mm, min via 0.3 mm drill). Fix all violations. | [ ] |
| G5 | Export Gerber files and upload to JLCPCB for a quote. Order 5 bare PCBs (~$15 for 5). Do not order assembly yet. | [ ] |
| G6 | When PCBs arrive, hand-solder components and flash firmware. Debug any hardware issues. | [ ] |

## Group H — Enclosure & Alpha Build (Week 12–16)

| # | Step | Done? |
| --- | --- | --- |
| H1 | Model the enclosure in Fusion 360 or FreeCAD. Export STL and 3D print 2 copies on an FDM printer (your own or a service like Craftcloud). | [ ] |
| H2 | Test fit: PCB, LCD, battery, and keypad dome sheet inside the printed shell. Adjust tolerances and reprint as needed. | [ ] |
| H3 | Assemble 3 complete alpha units. Use real LiPo batteries and verify charging over USB-C. | [ ] |
| H4 | Conduct a 2-week alpha test: daily use as an actual calculator. Log every bug, UI issue, and missing feature in a GitHub issue tracker. | [ ] |
| H5 | Fix critical bugs. Update firmware version to v0.9-alpha. | [ ] |

## Group I — Beta & Production Decision (Week 16+)

| # | Step | Done? |
| --- | --- | --- |
| I1 | Decide on funding path: self-fund 500 units (~$10,500 COGS) or launch a Crowd Supply / Kickstarter campaign to guarantee MOQ before committing. | [ ] |
| I2 | Source FCC/CE pre-certified ESP32-S3-WROOM-1 module (Espressif's module is already certified; confirm with supplier). | [ ] |
| I3 | Get quotes for injection-molded enclosure from 3 suppliers (Hubs, Xometry, or a China mold house). Mold cost is typically $800–$2,000 for ABS at this size. | [ ] |
| I4 | Submit JLCPCB PCBA order for 50 beta units with full BOM. Review assembled boards before placing 500-unit order. | [ ] |
| I5 | Run 2-week beta with 10–20 external testers. Collect feedback. Ship final firmware fixes. | [ ] |
| I6 | Place 500-unit production order. Set up GitHub Releases for OTA firmware updates. Launch. | [ ] |

Milestone to aim for: by the end of Group D you should have a single device on your desk running Python scripts that draw on an LCD screen. That is the moment the project becomes real.